# Draganflyer X6 Handheld Computerized Controller

## Size & Weight

- Width: 22cm (8.7in)

- Height: 12cm (4.7in)

- Depth: 8cm (3.1in)

- Weight: 895g (31.5oz)

## OLED Touch Screen Display

- Diagonal Size: 7.2cm (2.8in)

- Resolution: 240x320 px

- Colors: 65,536 (16-bit)

- Luminance: 160 cd/m2 (candela per square meter)

- Contrast Ratio: 10,000:1

- Viewing Angle: 170°

- Touch Screen Type: Pressure Sensitive Resistive

## Buttons & Controls

- 2 Dual-Axis Gimbals

- 3 Trim Tabs

- Shutter Button

- Ascent/Descent Switch

- Zoom In/Out Switch

- Variable Camera Tilt Knob

- GPS Position Hold Switch

- Trainer Button

- Touch Screen (Finger or Stylus Sensitive)

## External Connections

- Video Output:

  - Number of Video Output Jacks: 2

  - Connect To: Video Goggles & Recording Device

  - Output: Streaming Video Received From Helicopter

- Charge Input: Integrated Balance Connector and Power Connector

- SD Card Slot: Firmware Update, and Auxiliary Black Box

## Internal Rechargeable Battery

- Chemistry: Lithium Polymer

- Voltage: 11.1V

- Capacity: 2,000mAh

- Cell Configuration: 3s1p (3-series 1-parallel)

- Connectors: Integrated Balance Connector and Power Connector

- Recharge Time: 30 minutes

## Flight Simulator Controller Function

- Controller To Personal Computer Connection: Wireless

- Compatible With: FMS Flight Simulator

## Buddy Box Controller Function

- Main Controller to Assisting Controller Connection: Wireless

- Possible Configurations:

  - Flight Controls on Main & Camera Controls on Assist

  - All Controls on Assist & Full Override on Main

  - Custom Splitting of Controls Between Main and Assist

## Flight Recorder / Black Box

- Saves flight data to: on-board removable MicroSD memory card

- Use for: post flight analysis, troubleshooting, crash log

- Includes GPS Data

## Information Displayed

- Heading

- Bearing

- Altitude

- Climb Rate

- Speed

- Temperature

- Throttle

- X6 Helicopter Battery Voltage

- Controller Battery Voltage

- Data Link Quality

- Number of GPS Satellites Locked On

- GPS Reception Quality

- Attitude (Roll, Pitch, Yaw)

- Magnetometer Readings (Right, Forward, Down)

- Velocity (North, East, Down)

- GPS Latitude & Longitude

- Altitude (GPS & Barometric)

- GPS Accuracy & Position Errors

## Materials

- Rubber Grip

- ABS Plastic

## RF Control & Data Link

- 2.4GHz Data Link

  - Link Type: Helicopter to Ground & Ground to Helicopter (Two-Way)

  - Controller Antenna: Omni-Directional

  - Helicopter Antenna: Wired Whip Antenna

  - Transmission Power (North America): 100mW (+20dBm) EIRP

  - RF Data Rate: 250kbps

  - Receiver sensitivity: -100dBm

  - Transmission Technique: DSSS (Direct Sequence Spread Spectrum)

  - Frequency band: 2.4000 - 2.4835 GHz

  - Certifications: CE, FCC, IC, ETSI

  - Data Link Channel Selection: Automatic (12 Channels)

- Channels Supported: 16 (9 used, 7 free)

  - Throttle

  - Roll

  - Pitch

  - Yaw

  - Camera Shutter

  - Ascent

  - Camera Zoom

  - Camera Tilt

  - GPS Position Hold

## Embedded DraganEye™ Pro 5.8GHz Dual Diversity Wireless Video Receiver

- Model: DraganEye™ Pro

- Diversity Type: Dual

- Diversity Video Unit Switches Between Received Signals Based On:

  - Video Quality

  - RSSI Signal Strength

- Video Link

  - Frequency: 5.8GHz

  - Link Type: Helicopter to Ground (One-Way)

  - Receiver Antennas: Omni-Directional & Flat Patch

  - Transmitter Antenna: Omni-Directional

  - Transmission Power: 12dBm

  - Transmitter Power Consumption: 500mW

  - NTSC and PAL Compatible

  - 7 Selectable Channels: 5740MHz, 5760MHz, 5780MHz, 5800MHz, 5820MHz, 5840MHz, 5860MHz