EMERGENCY PROCEDURES:

In addition to basic mission plan waypoints, alternate (contingency) mission segments (branches) are associated with each waypoint.  Contingency routes provide the capability to return to base (RTB) autonomously or when manually commanded by the pilot.  An autonomous RTB would result from data link communications losses or failure of mission essential equipment and/or aircraft subsystems.

The RQ-4 Global Hawk flight management system is programmed for each route segment to autonomously perform a specific contingency mission profile in the event of specified anomalies or system/subsystem failures.  Global Hawk has three primary contingency modes: (1) Lost Data Link Communications, (2) Return to Base and (3) Emergency Landing.  Lost Link contingency (1) routes will always be designed to avoid special use airspace that has not been pre-coordinated and comply with standard TFR altitudes/routes.  The RTB contingency (2) initiates a three-minute countdown timer before returning to the departure airport or an alternate pre-coordinated airfield via a pre-planned route.  This delay affords the pilot time to coordinate a clearance to RTB and take steps that account for ATC assigned heading and/or altitude restrictions.  The Emergency Landing contingency (3) results in the aircraft being directed to an alternate, pre-coordinated airfield or nominal safe surface impact area (dry lake beds, unused military auxiliary fields, etc).

In the event of subsystem problems which jeopardize aircraft operational capability, the aircraft is programmed to autonomously return to the departure airport or a pre-selected alternate landing site.  The pilot will ensure that appropriate ATC facilities are notified of the emergency and the routing that the aircraft will fly whenever a contingency route is executed.  A list of telephone numbers will be prepared in advance for each ATC supervisory position responsible for airspace the aircraft is programmed to operate in.

If the emergency is flight critical requiring immediate recovery, the aircraft transponder will automatically change to code 7700.  The takeoff and mission can be aborted by either the LRE or MCE pilot or by the aircraft itself.  Takeoff aborts are initiated by the aircraft or a pilot directed Takeoff Abort command.  Takeoff abort is inhibited once the main gear weight-on-wheels sensors detect lift off at which point the aircraft initiates one of the three contingency plans previously described.  The aircraft can manually be sent around/waved off by the pilot in the LRE, MCE or autonomously by the aircraft itself.  Pilot directed go-arounds/wave offs are initiated by a Land Abort command.  In response to a Land Abort command, the aircraft executes a preplanned route of flight back to the approach runway where it will land if not sent back to the primary approach.  A Land Abort command is not possible when the aircraft is below 130 feet AGL or has started to flare.

See Guam RQ-4 arrival/departure, missed approach, emergency divert (NWF) and ditch point routing attached in the Flight Operations Area/Plan section.<<<<<<<