LAUNCH/RECOVERY:

The RQ-4 Global Hawk uses its INS/GPS system as the sole means to accomplish autonomous departures and self-contained approaches to landing.  Horizontal guidance is provided by the GPS.  Vertical glide slope guidance is provided by redundant radio altimeters or INS/GPS altitude.  <<<<<<<The aircraft starts to flare for landing between 150 and 24 feet AGL. The exact flare altitude varies with the approach rate of descent and normally starts about 40 to 70 feet AGL.  During the flare, the rate of descent is reduced to 2 feet per second and a decrab maneuver removes any crosswind correction.

Global Hawk cannot use existing Flight Information Publication approach procedures.  Approaches are built by the Air Force Mission Support System (AFMSS) mission planner and are individually validated within the AFMSS constraint check and an independent six-degrees-of-freedom simulation model.  Global Hawk flies a straight-in approach on a 4.5 degree glide path and does not use existing USAF approach procedures.  Approach waypoints in the mission plan have defined horizontal position and height requirements.  The RQ-4 autonomously executes the pre-planned approach to landing with no operator intervention required.  If the RQ-4 determines that it is exceeding preset cross-track or altitude criteria, the aircraft will autonomously execute a go-around maneuver.  The pilot can command a manual go-around based on ATC direction, ground safety observer input or observation of runway alignment from the Forward Looking Infrared (FLIR) nose camera.

See Guam RQ-4 arrival/departure, missed approach, emergency divert (NWF) and ditch point routing attached in the Flight Operations Area/Plan section.<<<<<<<