MITIGATION PROCEDURES (IAW JO7610.4, Chapter 12, section 9):

Primary see-and-avoid mitigation procedures at Andersen AFB (AAFB) will consist of Temporary Flight Restrictions (TFR) under Part 99.7.  The TFR will extend from the surface to the bottom of the day Class A airspace (20,000’ MSL).  Lateral TFR dimensions are depicted in an attachment in the Flight Operations Area/Plan section.  TFR lateral and vertical dimensions were coordinated with Guam FAA CERAP.

Additional mitigators include:

- Within Andersen AFB (AAFB), Surface Class D Airspace;

  - Control Tower observer (radar)

    - Tower observer maintains 2-way voice communications with RQ-4 Pilot in Command

  - Ground observer (visual)

    - Mobile observer located in “Hawkeye” vehicle

- Outside AAFB Class D, but within TFR and/or Class A airspace

  - Radar traffic advisories by FAA CERAP using ASR ATC radar

    - Standard IFR separation procedures