Attachment 2 Aircraft Type and Model Description

AeroVironment (AV) Wasp

The Wasp Small Unmanned Aircraft System (UAS) is a hand-launched reconnaissance and surveillance tool. The system transmits live airborne video images, compass headings, and location information to a Ground Control Station (GCS).  This enables operators to navigate, search for targets, recognize terrain, and record all information for analysis.

The Wasp vehicle can be launched and recovered in minutes without special equipment. The system employs a self-stabilizing vehicle configuration with stability augmentation avionics, which provides ease of control and steady video imagery. Air and ground components are lightweight. The Wasp vehicle is battery powered and has low visual, acoustic, and thermal signatures. The system includes one EO and one IR camera payload.  The Wasp vehicle flies for approximately 45 minutes on one rechargeable Lithium Ion (Li-ion) battery pack.  Wasp may be operated by a two person team consisting of a Vehicle Operator (VO) and a Mission Operator (MO).  A single operator performing both the VO and MO duties may operate the Wasp.

UMA AE SYSTEM DESCRIPTION

The Wasp vehicle is assembled from components. No TSO Components are used.  It is broken down for storage and transportation into one case containing fuselage, wing tips, rudders, and props.  Payloads and GCS are packed separately.  The assembled Wasp vehicle is shown in Figure 1 and 2.  Table 1 shows the vehicle characteristics.

Figure 1.  Wasp

( 28.5 inch x 9.3 inch )

Figure 2.  The Wasp Overall Dimensions

| Parameter | Characteristic |
| --- | --- |
| Payload | EO and IR cameras |
| Navigation | Global Positioning System |
| Range | 5 km line-of-sight (LOS) |
| Endurance | 45 Minutes |
| Speed | 20-40 mph |
| Operating Altitude | 150 to 1,000 ft AGL |
| Span | 28.5 inch |
| Length | 9.3 inch |
| Weight (with payload) | 16 ounces |
| Launch Method | Hand-launched |
| Recovery Method | Glide landing |

Table 1.  Wasp Vehicle Characteristics