Gadsen Police Department policy on the use of Unmanned Aircraft System (UAS)

- PURPOSE:

The Gadsen Police Department is implemented a small unmanned aircraft system (“sUAS”) program to assist law enforcement in providing situation awareness. This policy sets forth how the sUAS program will operate the aircraft in coordination with law enforcement officers conducting a specific mission. This policy is designed to minimize risk to aircraft, property and people during the operation of the sUAS.

- DEFINITIONS:

  - Mission Commander - The individual responsible for reviewing and approving the use of the sUAS in a law enforcement mission.

  - Observer – The individual trained to assist the pilot in command in carrying out all duties required for the safe operation of the sUAS.

  - Pilot in Command – The individual responsible for the overall flight operations of a specific mission. The pilot in command has final authority as to all flight operational issues, including the final go/no-go decision.

  - Program Manager - The individual responsible for managing all aspects related to the use of sUAS and Law enforcement mission.

- POLICY:

  - Aircraft:

    - Airworthiness – The Mission Commander shall be responsible for ensuring that the UAS is airworthy prior to each mission.  The Mission Commander may rely upon the inspection and reports provided by agency personnel responsible for maintaining the UAS.  In addition, the Mission Commander may rely upon the testing data and evaluation data provided by other government agencies, the aircraft manufacturer, and consultant.

    - Maintenance – All maintenance shall be performed by agency personnel specifically trained on the maintenance of the UAS or by manufacturer certified representatives and personnel.  The UAS will undergo a preflight and post flight inspection.  Any squawks or discrepancies will be entered in the aircraft’s log and reported to the sUAS program manager.  It shall be the responsibility of the sUAS program manager and Mission Commander to determine whether the reported squawks or discrepancies need to be corrected prior to the next flight.

  - Pilot:

    - All pilots who will be flying law enforcement missions shall be properly trained by either manufacturer representatives or individuals designated by the sUAS Program Manager.  The pilots will have a current working knowledge of the airspace intended for operations, air traffic control communication requirements, specific sUAS aerodynamic factors, and the ability to obtain and interpret weather.  All pilots must meet the currency requirements and must meet the following flight experience requirements:

      - Specific UAS Model Experience: All pilots must have at least 10 hours of flight time in the UAS model and 20 launch and recovery operations in the UAS model prior to flying the aircraft in a Mission.  In addition to basic aircraft operations, appropriate training shall include a review of communications, emergency procedures and mission limitations.  The Pilot shall also meet the currency requirements as set forth below.

      - Initial Mission Training: All pilots must undergo mission training.  Mission training is additional training beyond basic flight operations.  Mission training shall include:

        - Preflight preparation

        - Preflight briefing for observer

        - Communications between pilot and observer

        - Standard surveillance patterns

      - Surveillance Missions:  A pilot must have logged 20 flight hours as PIC before flying surveillance. An observer shall have logged at least 10 hours as an observer prior to acting as a primary observer on a surveillance missions.

      - Search & Rescue Missions:  A pilot must have logged 20 flight hours as PIC before flying search & rescue. An observer shall have logged at least 10 hours as an observer prior to acting as a primary observer on a search & rescue missions.

      - Prohibited Acts:

        - Exceeding Aircraft Limitations:  The sUAS shall not be flown in conditions that exceed the manufacturer’s recommended limitations, including range, ceiling, and wind strength.

        - High Risk Missions: The sUAS shall not be flown for any mission that any mission participant determines that the risk of flying the sUAS outweighs the benefit to the mission.  Risk may include hazards to individuals or property on the ground, possible collision hazard with other aircraft, loss of control of the sUAS.  If a mission develops into a high risk mission, the mission shall be terminated by the PIC.

    - Currency Training:

      - Pilots:  All pilots must have 3 flight events with launch and recovery in the proceeding 90 days in UAS model to be eligible for a law enforcement flight mission.

Each pilot must undergo re-currency training every 12 months. Re-currency training shall include:

        - Assembly and Disassembly

        - Launching and Recovery

        - Review of Relevant FAA Regulations

        - Emergency Operations

        - Low Altitude Operations

        - Manual Operations

        - Surveillance Patterns

        - Weather

  - Observer:

    - Observer Required: An observer is required for all missions.

    - Observer Experience: All observers must participate in the preflight briefing, which shall include but not be limited to a review of mission goals and methods to accomplish those goals, review of communication procedures, and a review of emergency procedures. The observer will receive specific training on relevant Part 91 regulations, such as the obligation to see and avoid other aircraft and the ability to identify position for purposes of relaying position reports to other types of aircraft.

  - Operational Conditions:

    - Line of Sight:  All operations shall be conducted within line of sight of the Pilot or Observer such that the Pilot or Observer may detect and avoid hazards such as property and aircraft.

    - Weather:  All flights will be conducted in VFR weather with the informant amounts consistent with Class E airspace minimums.  Flight may not be conducted in known icing conditions.

- FLIGHT REQUIREMENTS:

  - Mission Request: all requests for sUAS to provide support for a long portion mission shall be afforded to the Program Manager. The Program Manager shall discuss the proposed mission with the Mission Commander.  Such discussion will consider the following:

    - The location of the mission for purposes of insuring the safety of people and property;

    - The intended area of operation for purposes of evaluating the ability to mitigate potential air to air conflicts. Such evaluation will take in consideration the current landing patterns at airports in the vicinity. Whenever the approach path of an airplane to a nearby airport would involve flying over the intended area of operation, such operations shall be coordinated with the appropriate air-traffic control facility.

    - The weather and its potential effect on the aircraft, including the ability to carry the aircraft to potential area of high air to air conflict.

    - The currency of the proposed pilot and observer.

    - The potential usefulness of the information gathered by the sUAS versus information gathered through other means.

    - Any other relevant risk factors to successfully complete a risk benefit analysis for the use of sUAS in the long portion mission.

  - Once the Program Manager has approved the mission request, the Mission Commander's identify the pilot and observer and coordinate with individuals requesting the mission.

  - Preflight Preparation:  Before any mission the pilot must conduct a preflight briefing (See Attached Form).  The briefing shall include the following:

    - Identification of mission purpose.

    - Identification of individuals participating in the mission regardless of whether they are aircraft or ground support.

    - Identification of mission limitations, including weather limitations and flight time limitations.

    - Identification of flight area, including maximum ceiling and floor.

    - Coordination of communication procedures between Pilot and Observer and Aircraft and ground support.

    - Emergency Operations including review of aircraft system failure and lost link.

  - Scene Review: The Pilot and Safety team is responsible for securing and identification of all safety items at the scene.  These include:

    - Take-off and landing site: This area needs to be free of obstructions, items on the ground and debris that may interfere with the rotors. This includes creation of a flight line, which other law enforcement officers and civilians must stand back from this area.

    - Flight perimeter: The site must utilize observers, law enforcement officers and standard protocols to minimize civilian traffic or interference during the operation.

    - Safety View: The flight team should identify trees, bushes, power lines, and other potential obstructions and coordinate the preflight briefing accordingly.

    - Interference: The flight team should identify Cell Towers, TV and Microwave sources, which might create interference with the flight equipment.  The equipment should be tested on the ground to insure proper communications and operation before the flight.

  - ATC notification:  The pilot shall notify the appropriate air-traffic control facilities in advance as early as possible. Such notification should include the following:

    - The intended location of the flight,

    - The intended duration of the flight,

    - The maximum altitude of the flight, and

    - The cell phone of two individuals for emergency contact.

    - The pilot shall notify air-traffic control immediately at the conclusion of the flight.

  - Communications:  The pilot and/or the observer shall have a VHF radio capable of transmitting position reports to nearby aircraft. It shall be the responsibility of the pilot and/or the observer to monitor local communication frequencies to assist in the de-conflicting the sUAS from other aircraft.

  - Flight operations: All flight operations shall be conducted in accordance with the manufacturer's manual. If any time the pilot and/or observer believe there is a potential for air to air conflict, risk of harm to individuals or property, the pilot shall immediately landing aircraft. In the event of loss communications with the aircraft, aircraft shall fly to a predetermined location and land. If the aircraft does not immediately execute these orders, the pilot or observer shall immediately notify air-traffic control.

- REPORTING:

  - The Mission Commander shall be responsible for logging all flights. The flight log should include the following:

    - The name of the pilot,

    - The name of the observer,

    - The area of operations,

    - The duration of the flight, and

    - Any unusual circumstances (including loss of link or aircraft damage).

  - Reporting incidents or accidents: the Mission Commander shall also be responsible for reporting any incidents or accidents to the local Flight Standards District Office.