Attachment 7 Emergency Procedures

Preventing vehicle loss or damage depends on early recognition of dangerous flight conditions or malfunctions, followed by appropriate corrective action.

Accurate decision-making depends on a through understanding of system operation and behavior.  Crew effectiveness in an emergency requires preparation.  Mission planning includes alternative courses of action available for each phase of the planned mission.

Operators and the Pilot-in-Command (PIC) continually monitor the vehicle data in order to notice anomalies as soon as they develop.  During flight, the operators and PIC maintain situational awareness, i.e. altitude, flight time, battery voltages, waypoint data, warnings, range, bearing, and antenna orientation.  The VO should always know which direction to fly to escape a hazard.  The MO should always know the vehicle position relative to hazards and be ready to give the VO headings and altitudes to fly the vehicle to safety.

The steps that must be performed immediately in the following emergency procedures are underlined and in bold print.  Non-underlined steps can be accomplished as time and the situation allows.

ITEMIZED EMERGENCY’s and ASSOCIATED PROCEDURES

- Loss of Downlink

If the link is not restored after the following emergency procedures the VO should perform a landing under their control when possible.

  - Check orientation of the downlink antenna

  - Command turn and climb

  - If link does not return, switch the GCS Tx OFF

    - This will force LOL mode, which will be set to return to predetermined location and land.

    - If the downlink video returns, switch TX ON and resume the mission.

- Loss of Component in Flight

  - Command Autoland

- Extreme Low Vehicle Battery

  - Switch to MAN mode

  - Select suitable landing area

  - Command Autoland

- Loss of Uplink

  - Monitor progress in the LOL.  Ground control will be set so that aircraft will fly to a specific location at a predetermined safe altitude and then execute a landing to a predetermined location.

  - If link returns, resume the mission.

- GPS Failure

  - Fly back azimuth or with terrain navigation to a recovery site or continue the mission in a degraded mode using the compass and land navigation techniques.

- Vehicle Operator Hand Controller Fails

  - Disconnect the hand controller

    - This will force LOL mode.

  - Replace with a spare hand controller.

  - Verify all settings are as desired then resume the mission.

- Steady Turn Develops at Neutral Stick

  - Return to landing area if able.

  - Command Autoland

- Vehicle Oscillates Up/Down or Right/Left

  - Return to landing area if able.

  - Command Autoland

- Loss of Downlink Data Only (Video Functioning)

  - Turn transmitter off

    - This will force LOL mode

  - Monitor the vehicle location

  - Recover the vehicle

- Altitude Hold Failure

  - Enter MAN mode

  - Fly back to recovery site or continue mission in degraded mode

    - Control altitude with 40-50% power

    - Assist with joystick as needed

When recovering the aircraft record all the last good vehicle data before disconnecting the GCS power.  If flight is being recorded, make notes of operator’s last actions and last observation of the vehicle video.  Save video recording for review.

The vehicle battery usually does not disconnect on landing; therefore vehicle may continue to transmit a downlink signal.  The vehicle payload may disconnect on landing, in which case the video image may be black, not snow.